Task 11—Monitor and Control the Injection Rate of the Corrosion Inhibitor

1.0	Task Description

The task consists of monitoring and controlling the injection rate of corrosion inhibitor to mitigate internal corrosion.

This task begins with a visual observation of the injection system. The task ends when proper documentation and notification are completed.

2.0	Knowledge Component

The purpose of this task is to monitor corrosion inhibitor injection rates and adjust these rates to ensure that the proper amount of inhibitor is being injected.

An individual performing this task shall have knowledge of:

- Different types of chemicals and their injection rates

- How to calculate the formulas for injection rates

- Types of injections rates, such as continuous or batch injections

Terms applicable to this task:

corrosion

The chemical, electrochemical, or bacteriological reaction between a material, usually a metal, and its environment that produces a deterioration of the material and its properties.

inhibitor

A chemical substance or combination of substances that, when in proper concentrations, forms an environment that prevents or reduces corrosion.

mils per year

MPY

The rate of corrosion measured in 0.001 in. per year.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Damage or malfunction to injection system causing leak. | Stop task activities and make appropriate notifications. |
| Component failure, contamination, or other damage of injection system. | Adjust injection rate per operator or manufacturer’s specifications. If repair is required, Follow the operator’s policies/procedures for appropriate documentation, notification protocol, and actions required. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Visually inspect tank and injection system to verify that the injection system is operating and inspect for signs of leakage. | Verifies operation and integrity of the system. |
| 2 | Verify that the volume of inhibitor is sufficient to last until the next inspection. | Verify appropriate injection rates with operator’s personnel. |
| 3 | Monitor inhibitor injection rate to determine if the inhibitor rate meets the requirements based on the pipeline flow rate. | Follow the operator’s policies and procedures to determine the flow of the inhibitor. This may include measuring changes in the volume of inhibitor using a sight glass. |
| 4 | Adjust injection rate as necessary to meet established pipeline flow rate. | Follow the operator’s policies and procedures when adjusting the injection rate of the inhibitor. |
| 5 | Complete documentation, and make notifications per the operator’s procedures. | Follow the operator’s policies/procedures for appropriate documentation, notification protocol, and actions required. |