Task 121 - Underwater – Install and Remove a Plugging Machine

1.0 Task Description

This task involves installation and operation of a plugging machine to allow for isolation of a section of a pipeline and for removal of the plugging machine. This task is similar to Task 40.6 (Install and Remove Plugging Machine) but contains steps or equipment that are unique to an underwater environment.

This task begins with surface preparation for installation of a plugging machine. This task ends with removal of the plugging machine.

The performance of this covered task may require the performance of other covered tasks such as:

- Underwater – Install and Remove a Tap on a Pipeline System (reference Task 122)

This task does not include but may lead to the performance of other covered tasks such as:

- Underwater – Install and Remove a Completion Plug on a Pipeline System (reference Task 123)

2.0 Knowledge Component

The installation and removal of the plugging machine allows for isolation of a pipeline segment or the diversion of flow on an active pipeline.

An individual performing this task must have knowledge of:

- The details of the line segment, such as pressure, flow, pipe size, wall thickness, and product, to ensure that all materials (machine, fitting(s), appurtenances) are appropriately rated.

- A plugging machine is installed on an operating pipeline to temporarily isolate a section of the pipeline.

- The plugging machine serves as a temporary block valve.

- The notification(s) that must be completed prior to and/or after the task is completed, according to relevant Operator procedures.

Terms applicable to this task.

Containment equipment

Equipment such as containment domes, diaphragms, pumps, hoses, and receiving tanks (MPT Tanks) to capture hydrocarbons.

Completion plug

A plug designed to seal the opening created by a hot tap. The plug will allow installation and removal of the tapping machine or plugging machine and valve.

Fitting

A component welded or clamped to the pipeline upon which a valve is installed to allow tapping and plugging.

Plugging machine

A machine installed onto a valve for the purpose of inserting a plug to isolate a pipeline segment or divert the flow.

Tapping valve

The component installed on the fitting to control product flow while inserting the boring bar and operating the cutter on the pipeline or breakout tank and during removal of the tapping machine.

Abnormal operating conditions (AOC) associated with the performance of this task include:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Damage (e.g. mechanical damage or corrosion) to pipeline facilities or components. | Make appropriate notifications and follow applicable operator procedures. |
| Malfunction of or damage to tapping machine, tapping valve, or other related equipment that has the potential for loss of product. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Pipeline pressure exceeds the rated capacity of the plug. | Stop operation and secure the equipment, if safe to do so. Immediately notify the operator or appropriate individual. Inspect the equipment and readjust or reset, as necessary. |
| Inadequate supports for the plugging machine causes stress and pipeline damage. | Stop operation and secure the equipment, if safe to do so. Immediately notify the operator or appropriate individual. Inspect the equipment and readjust or reset, as necessary. |

3.0 Skill Component

To demonstrate proficiency of this task, an individual must perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Install the plugging machine and other fittings and appurtenances as required by the manufacturer’s specifications. | This step allows for the insertion of the plug into the pipeline and for pressure equalization. This will ensure valve and fitting will maintain integrity and prevent leakage. Verify that fittings and appurtenances have the required pressure rating. |
| 2 | Install the appropriate sandbag support for the plugging machine as necessary. | This step ensures that the weight of the plugging machine does not overstress the pipe. |
| 3 | Prior to opening the valve, equalize the pressure on each side of the valve if possible. | Equalized pressure facilitates operation of the plugging machine. |
| 4 | Set up containment equipment, conduct pressure test, and perform a visual or tactile inspection to confirm integrity of the plugging machine. |  |
| 5 | Slowly open the valve fully on the fitting. | This step allows the plugging machine access to the pipe. |
| 6 | Operate the plugging machine to insert the plug into place. | Plug insertion will stop the product flow. |
| 7 | Monitor the pipeline pressure upstream and downstream of the plug. | This step ensures that pressure does not exceed manufacturer’s specifications. |
| 8 | Bleed off the pipeline downstream of the plug to confirm that the plug is holding. | The plug may need to be reset if the pipeline does not bleed down to zero. |
| 9 | Confirm that maintenance repairs are complete. | — |
| 10 | Equalize the pressure on either side of the plug. | Equalized pressure will allow retraction of the plug. |
| 11 | Retrieve the plug from the pipe. | The plug is retracted into the plugging machine. |
| 12 | Close the tapping valve and relieve the pressure from the plugging machine. | The plugging machine is isolated from the product flow. Ensures that the tapping valve is fully closed. |
| 13 | Drain the plugging machine before removal. | Depressurize and drain the product trapped between the valve and plugging machine to ensure that residual product is not inappropriately released. |
| 14 | Remove the plugging machine from the tapping valve. | Install the cap, blind flange, piping, instrumentation, or other component onto the fitting or valve per the operator’s procedures. |
| 15 | Make notifications per the operator’s procedures. | Follow the operator’s policies and procedures for appropriate documentation, notification protocol, and actions required. |