Task 122 - Underwater – Install and Remove a Tap on a Pipeline System

1.0 Task Description

This task involves safely hot tapping into an operating pipeline. This task is similar to Task 40.8 (Install and Remove a Tap Larger Than 2 in. on a Pipeline System) but contains steps or equipment that are unique to an underwater environment.

This task begins with assembly of tapping machine, valve, and clamp. This task ends with the removal of the tapping machine from the valve and retrieval of the coupon.

The performance of this covered task may require the performance of other covered tasks such as:

- Measure Wall Thickness with Ultrasonic Meter (reference Task 8.2)

- Perform Welding (reference Task 42.7)

This task does not include but may lead to the performance of other covered tasks such as:

- Underwater – Install and Remove a Plugging Machine (reference Task 121)

- Underwater – Install and Remove a Completion Plug on a Pipeline System (reference Task 123)

2.0 Knowledge Component

Hot tapping is performed on an in-service pipeline to make connections without having to shut down.

An individual performing this task must have knowledge of:

- The details of the line segment, such as pressure, flow, pipe size, wall thickness, and product, to ensure that all materials (machine, fitting(s), appurtenances) are appropriately rated.

- Equipment to prepare the surface such as hydraulic buffers, high-pressure water blasters, or draw knife. Other equipment such as hydraulic saws, chippers, and grinders can damage the pipeline.

- The notification(s) that must be completed prior to and/or after the task is completed, according to relevant Operator procedures.

Terms applicable to this task.

Containment equipment

Equipment such as containment domes, diaphragms, pumps, hoses, and receiving tanks (MPT Tanks) to capture hydrocarbons.

Bleeder valve

A valve that allows the controlled relief of pressure.

Boring bar

The main shaft of a tapping machine that turns the cutter.

Coupon

The piece of wall cut from a pipeline or breakout tank with a cutter.

Cutter

The bit that cuts a coupon from the wall of a pipeline or breakout tank. The cutter is equipped with a pilot bit to bore a hole that will center the cutter.

Fitting

A component welded or clamped to the pipeline upon which a tapping valve is installed to allow tapping and plugging.

Hot tap

The process of safely cutting or boring a hole into an in-service pipeline or breakout tank.

Tapping machine/drilling machine

A machine installed onto the appropriate tapping valve for the purpose of boring a hole into a pipeline or breakout tank

Tapping valve

The component installed on the fitting to control product flow while inserting the boring bar and operating the cutter on the pipeline or breakout tank and during removal of the tapping machine.

Abnormal operating conditions (AOC) associated with the performance of this task include:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Damage (e.g. mechanical damage or corrosion) to pipeline facilities or components. | Make appropriate notifications and follow applicable operator procedures. |
| Malfunction of or damage to tapping machine, tapping valve, or other related equipment that has the potential for loss of product. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Unexpected release of hazardous liquid. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Lost coupon may damage downstream equipment. | Immediately notify the operator. |
| Pressure trapped between the tapping valve and the hot tap machine. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Loss of product by boring hole through opposite wall of pipeline. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Improper alignment of the valve can result in damage to the valve or tapping machine. | Stop operation and secure the equipment if safe to do so. Inspect the equipment and readjust or reset as necessary. |
| Damage to pipeline from inadequate support of tapping machine that could affect integrity of the pipeline. | Stop operation and secure the equipment if safe to do so. Determine the extent of the damage and make appropriate notifications and documentation as required by the operator. |
| Pipeline pressure exceeds the rated capacity of the tapping machine. | Stop operation and secure the equipment, if safe to do so. Immediately notify the operator or appropriate individual. Inspect the equipment and readjust or reset, as necessary. |

3.0 Skill Component

To demonstrate proficiency of this task, an individual must perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Verify the proper tapping valve and the clamp are appropriate size and rating for the application to be completed. | This will ensure tapping valve, tapping machine size, rating, and clamp are fit for purpose for the hot tapping task. |
| 2 | Prepare tapping machine, valve, clamp, and rigging to ensure the cutter can pass through the open tapping valve and the tapping valve can be fully closed when the cutter is retracted. | Topside personnel and divers involved to ensure proper assembly, operation, and isolation according to appropriate manufacturer’s and operator specifications. |
| 3 | Take necessary measurements to determine the depth of cut. | Accurate measurement is important to ensure the tap is performed correctly. |
| 4 | Verify go/no go operating conditions meet operator procedures and requirements. | Ensure that pressures, level, product, weather conditions, or other operational parameters are safe to proceed with hot tapping task. |
| 5 | Determine the approximate location of the pipeline section, by using the most current drawings, maps, and/or GPS. | Drawings and/or pipeline maps are used to assist in locating the pipeline. |
| 6 | Prepare pipeline surface according to manufacturer’s and operator specifications. |  |
| 7 | Perform visual or tactile inspection to verify that the pipeline surface has been prepared according to manufacturer’s and operator specifications. | Proper preparation is required to ensure that the longitudinal weld seam is removed and sealing surfaces are cleaned to bare metal. |
| 8 | Verify wall thickness and check ovality and linear straightness for appropriate tolerance. | This step ensures appropriate wall thickness and ovality prior to installation of the tapping machine. |
| 9 | Install tapping clamp assembly and supports in proper orientation. | This step ensures that the tapping assembly clamp is safely lowered, supported, and installed on the pipeline according to appropriate procedures. |
| 10 | Conduct leak test. | Testing requirements are determined by operator procedures. Visual or tactile inspection to confirm that no leaks occur. |
| 11 | Operate tapping machine to complete hot tap according to manufacturer’s instructions. | Verity depth measurements of cutter assembly during hot tapping process to prevent drilling through the opposite pipe wall. |
| 12 | Raise the boring bar, cutter, and pilot bit and verify depth measurements to ensure valve clearance and allow closure. | Valve closure is necessary to prevent product release. |
| 13 | Close the tapping valve. | Prevent release of product when tapping machine is removed by ensuring that the tapping valve is fully closed. |
| 14 | Set up containment equipment. | This step is performed to capture any product remaining in the annulus. |
| 15 | Depressurize and flush the product trapped between the tapping valve and the hot tap machine if applicable. | Prepare for removal of the tapping machine. |
| 16 | Remove the tapping machine. |  |
| 17 | Confirm retrieval of coupon. | Provide to appropriate personnel for evaluation. |
| 18 | Make notifications per the operator’s procedures. | Follow the operator’s policies and procedures for appropriate documentation, notification protocol, and actions required. |