Task 124 - Underwater – Perform Flange Bolting

1.0 Task Description

This task involves connection of flanges applying the appropriate bolting sequence and torquing specifications.. This task is similar to Task 40.13 (Perform Flange Bolting) but contains steps or equipment that are unique to an underwater environment.

This task begins with the identification of the installation location. This task ends with the completion of appropriate documentation.

The performance of this covered task may require the performance of other covered tasks such as:

- Install Bonds (reference Task 9.1)

- Install Electrical Insulating Device – Piping Isolation (reference Task 9.6.1)

2.0 Knowledge Component

The purpose of this task is to properly align two pipe segments with flanges.

An individual performing this task must have knowledge of:

- Operator approved equipment, including gasket types and lubricants;

- Flange types and appropriate applications

- Lockout/tagout procedures;

- Torquing equipment and operator procedures;

- Flange class ratings.

Terms applicable to this task.

Pneumofathometer

A depth-measuring device consisting of an open-end hose fixed to the diver, with the surface end connected to a gas supply and pressure gauge (usually marked in Feet of Seawater or FSW). The gauge measures the pressure required to discharge water to the diver’s depth.

Drift pins

Tapered steel rod used to align flanges.

Lacing slings

A thin flexible sling between bolt holes to draw the flanges together during alignment.

Flange skillet

A carbon steel, stainless steel, aluminum, or synthetic ring that is used to center the o ring in the flange.

RTJ

Ring Type Joint

Abnormal operating conditions (AOC) associated with the performance of this task include:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Discovery of damage on the flange faces, gasket, or ring. | Make appropriate notifications and follow applicable operator procedures. |
| Flange misalignment or pinched o-ring | Stop task activities. Determine the cause of the misalignment and make corrections to align properly. |
| Unexpected release or discharge of product. | Stop task activities, eliminate ignition source(s), and notify appropriate personnel. |
| Improperly installed components | Remove defective or improperly installed bolts or gasket(s). Replace and install according to manufacturer’s specifications. |
| Debris in flange faces if excavation is not maintained | Remove debris and maintain excavation with air lift or hand jetting. |

3.0 Skill Component

To demonstrate proficiency of this task, an individual must perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Identify the location for flange installation. | This step ensures that work is being completed in the correct location and in accordance with the operator’s design plan. |
| 2 | Inspect flange bolting equipment and materials. | This step ensures that the proper tools, material selection, and equipment are serviceable and calibrated. This ensures that materials are inspected for proper sizing, damage or corrosion, gasket type, bolt and nut length and diameter, and flange assembly (rating and size) based on the series of flange. |
| 3 | Excavate area around flange to enable the installation tool usage | This excavation needs to be maintained using air lift or hand jetting throughout the process to prevent sediment or debris from accumulating during flange assembly. |
| 4 | Clean the flange faces, nuts, gaskets, studs, and bolts, as necessary. | Verify that rust, debris, marine growth, or other contaminants are removed from the flange faces and ring grooves with a wire brush or other approved cleaning method. Gaskets, nuts, and bolts will be new hardware according to operator specifications. |
| 5 | Align the flange faces. | Insert drift pins and lacing slings if needed to align the flanges. |
| 6 | Place two bolts in the lower portion of the flange and install an approved gasket. | This step prevents the o-ring (RTJ) from falling out. Flange skillets may be used to help center the o-ring. |
| 7 | Remove drift pins and lacing slings. Install all or remaining bolts and hand tighten the nuts. | Hand tighten all nuts until proper thread engagement is achieved. This helps maintain proper alignment and capture the o-ring prior to full torque. |
| 8 | Bolt up the flange and tighten to specified torque according to operator’s and manufacturer’s procedure. | Verify the proper flange tightening sequence and procedure according to the operator’s procedure. Pay close attention to angularity and offset to ensure that the flange faces are parallel (same gap around entire flange). Bolts or stud bolts shall extend completely through the nuts. |
| 9 | Perform a visual or tactile inspection of the completed flange. | Inspect the gap alignment as applicable. |
| 10 | Make notifications per the operator’s procedures. Complete required documentation. | Complete appropriate documentation according to the operator’s procedures. |