Task 141 - Underwater – Move In-Service Pipe

1.0 Task Description

This task consists of the movement of in-service pipe. This task is similar to Task 33 (Move In-Service Pipe) but contains steps or equipment that are unique to an underwater environment.

This task begins with identification of the pipe segment to be moved. This task ends when all notifications and documentation have been completed.

The performance of this covered task may require the performance of other covered tasks such as:

- Underwater – Examine for Mechanical Damage (reference Task 102)

- Underwater – Examine for External Corrosion (reference Task 103)

- Underwater – Inspect the Condition of External Coating (reference Task 104)

- Observe Excavation Activities (reference Task 32)

- Perform Backfilling (reference Task 39)

2.0 Knowledge Component

An individual performing this task must have knowledge of:

- Pressure restriction requirements;

- Soil conditions;

- Maps and drawings;

- Supports;

- Rigging equipment

- The details of the line segment, such as pressure, flow, pipe size, wall thickness, and product.

Terms applicable to this task.

Pneumofathometer

A depth-measuring device consisting of an open-end hose fixed to the diver, with the surface end connected to a gas supply and pressure gauge (usually marked in Feet of Seawater or FSW). The gauge measures the pressure required to discharge water to the diver’s depth.

Jetting

A diver handheld tool to deliver low pressure high volume water for excavation.

Air lift/Water lift

A tool used to assist in jetting operations by removing soil using air or water and the venturi effect.

Davit

A mechanical lifting device.

Helical pile/Bridge supports

A pile system to provide support to open spans of a pipeline.

Lift bag

A parachute or pillow type inflated to move heavy objects.

Abnormal operating conditions (AOC) associated with the performance of this task include:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Discovery of damage (e.g. mechanical damage or corrosion) on pipeline facilities or components. | Make appropriate notifications and follow applicable operator procedures. |
| Unexpected release or discharge of product. | Stop operation. Immediately notify the operator and execute applicable emergency procedures. |
| Component malfunctions or fails. | Stop task activities and make appropriate notifications. |

3.0 Skill Component

To demonstrate proficiency of this task, an individual must perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Determine the location of the pipeline section, by using the most current drawings, maps, and/or GPS. Review the engineering design plan or work scope indentifying the correct line, product type, location, and trench lengths. | Verifies that the correct line is being moved and the new location does not encroach on other existing pipelines. This step assumes that a permit has already been obtained. |
| 2 | Perform visual or tactile inspection for damage or repairs and determine if it is fit to move. Review historical repairs and pig readings. | This step is performed to prevent accidental release of product during the moving process. |
| 3 | Review the approved rigging plan for support requirements during movement (e.g. davit, lift bags, spreader bars). | To prevent over-stressing, buckling, excessive bend radius, or other issues. |
| 4 | Ensure rigging is installed at appropriate locations and that the pipe is supported in accordance with the approved rigging plan. | The design could determine the trench length calculation and maximum allowable support spacing. Chaffing gear will be installed on any riggging that is in contact with bare pipe. |
| 5 | Ensure the approved rigging plan is communicated with personnel on-site prior to beginning to move the pipe. | This ensures that the movement is synchronized and does not cause undue stress to the pipe. |
| 6 | Notify the control center that movement is about to begin. | The approved rigging plan will include instructions for the control center to reduce pressure, isolate the line, or take actions prior to moving the pipe. |
| 7 | Begin moving the pipe until the desired location has been obtained. |  |
| 8 | Remove the temporary pipe supports (rigging). |  |
| 9 | When the pipeline is in it’s final position, determine if additional support is needed and verify proper depth of cover if applicable. | This step is to confirm that the pipe has the correct supports (e.g. natural bottom, bridge support, helical piles, and sand cement) and remove any debris if found. Hand jetting may be needed to obtain appropriate pipeline depth. |
| 10 | After the move is complete, perform a visual or tactile inspection for damage. |  |
| 11 | Make notifications, as required by the operator, to the control center and affected personnel. |  |
| 12 | Document all required information per the operator’s procedure. |  |