Task 2.1—Verify Test Lead Continuity

1.0	Task Description

This task consists of the electrical inspection of test leads connected to a structure.

This task begins with identification of the test lead wire. This task ends when a determination is made about whether valid data may be obtained using the test lead wire.

The performance of this covered task may require the performance of other covered tasks such as:

--   Measure Structure-to-soil Potentials (reference Task 1.1).

This task does not include but may lead to the performance of other covered tasks such as:

Repair or Replace Damaged Test Lead (reference Task 2.2);

Install Test Leads by Nonexothermic Welding Methods (reference Task 2.3);

Install Test Leads by Exothermic Welding Methods (reference Task 2.4).

2.0	Knowledge Component

The purpose of this task is to test for electrical continuity between a structure and test station.

An individual performing this task shall have knowledge of:

- interpretation of structure-to-soil (electrolyte) potential measurements taken at a test station [which may not meet expected results (lower than anticipated, unstable, or erratic) and may be indicative of a broken test lead];

- multimeters (which are used to measure resistance between a structure and a test-lead wire to determine if continuity exists).

Terms applicable to this task:

data logger

A digital device used to record multiple structure-to-soil potentials.

reference electrode

Another term for a half cell or reference cell.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Abnormal or erratic readings on test equipment. | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Missing or damaged test points, leads, or stations. | Make the appropriate notifications for repair or replacement. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Identify the test lead to be tested. | This step is to confirm that measurements are taken on the intended test lead. |
| 2 | Select the proper equipment (multimeter, data logger, reference electrodes, etc.) to be used and verify the proper operation. | Damaged, incorrect, or faulty equipment will not provide accurate results and shall be repaired, replaced, or calibrated, as required. Make appropriate notifications if there are missing, damaged, or malfunctioning components. |
| 3 | Connect the test equipment to the structure to perform the test, as required. | Improper connection of equipment will lead to inaccurate potential or continuity measurements. |
| 4 | Measure the structure-to-soil (electrolyte) potential and/or continuity. | This step determines the potential and/or continuity of the structure and test lead. A potential may be compared with historical data to determine continuity. If test lead wire, test points, and/or test stations are damaged, missing, or loose, implement mitigation measures per the operator’s procedures. Measuring structure-to-soil potential is covered under another task (Task 1.1). |
| 5 | Document all required information per the operator’s procedures. | Up-to-date records are essential for maintaining a corrosion control system. |