Task 3—Obtain a Voltage and Current Output Reading from a Rectifier

1.0	Task Description

This task consists of measuring and documenting the electrical output of a rectifier.

The task begins with the identification of the rectifier. This task ends with the measurement of a rectifier output and documentation of data.

This task does not include data analysis.

This task does not include but may lead to the performance of other covered tasks such as:

- Troubleshoot Rectifier (reference Task 4.1).

2.0	Knowledge Component

The purpose of this task is to verify the proper performance of a rectifier.

An individual performing this task shall have knowledge of:

- cathodic protection (CP) systems and components comparable to AMPP/NACE Certification Level CP 1;

- voltmeters;

- clamp-on ammeters;

- calculating current from shunt factor and voltage measurement [current output may be calculated based on shunt factor (ratio) and voltage drop across the shunt];

- validation of display meters with observed readings and with remote read devices, if applicable;

- proper rectifier output polarity.

Terms applicable to this task:

This section intentionally left blank.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Reading outside of expected parameters, such as reverse polarity or inoperable rectifier. | Notify appropriate personnel of the observed condition for further analysis and repair, if required. |
| Energized rectifier cabinet. | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Identify the rectifier. Verify the rectifier cabinet is not energized. | Verifying that the rectifier cabinet is not energized is critical for personnel safety. |
| 2 | Determine the voltage by connecting a voltmeter across the output terminals of the rectifier. - Connect the positive lead to the rectifier positive terminal. - Connect the negative lead to the rectifier negative terminal. | Obtaining accurate voltage and polarity are essential to maintaining CP. If the polarity is reversed, take action, if qualified, and make appropriate notifications per the operator’s procedures. |
| 3 | Obtain the shunt factor by reading the value labeled on the shunt and dividing the amp value by the mV value. | Obtaining a shunt factor is essential to calculate current from millivolt reading obtained from a shunt. |
| 4 | Determine the current on a pre-installed shunt by reading the millivolt drop across the shunt and multiplying by the shunt factor. | Obtaining accurate current is essential to determining the effectiveness of a CP system. If the rectifier is inoperable, make appropriate notification per the operator’s procedures. |
| 5 | Check voltage and current readings against display meters and/or remote monitoring devices, if applicable. | Validating remote devices and display meters is necessary to ensure that accurate data are being received. |
| 6 | Document all required readings per the operator’s procedures. | Documents are essential to maintaining a corrosion control system. |