Task 15.1—Perform Visual Inspection of Surface Conditions of Right-of-way

1.0	Task Description

This task consists of performing an inspection of surface conditions in, or adjacent to, the pipeline right-of-way. Methods of inspection may include walking, driving, aerial, or other appropriate means of traversing the right-of-way.

The task begins with accurately identifying the right-of-way to be inspected. The task ends with completion of the required documentation.

2.0	Knowledge Component

The purpose of the inspection is to identify and observe for indications of leaks, construction activity, and other factors affecting safety and operation.

An individual performing this task shall have knowledge of:

- immediate threat to persons, property, or the environment;

indications of a release:

  - vapor cloud or frost ball on or near a right-of-way;

  - sheen on or bubbles in the water on a right-of-way;

  - dead vegetation or wet spot;

  - odor of hazardous gas or liquids;

  - audible cues (hissing, roaring, etc.);

  - fire on the right-of-way;

exposure that could immediately damage or affect the stability of a pipeline;

fire or explosion on or near the right-of-way.

Conditions that could impact the safety or integrity of the pipeline include the following:

- construction or excavation equipment or other signs of construction activity on or near a right-of-way;

- soil movement, such as a landslide, mudslide, sinkhole, subsidence, or settling;

- sagging aboveground pipe at a span;

- damaged, leaning, or failing pipe support system;

- unusual materials, equipment, and foreign objects on or near the right-of-way;

- damage to pipeline facilities or suspicious activity that might indicate vandalism or unlawful actions.

Conditions that could impact the pipeline, resulting in a response that would include reporting or maintenance:

- vegetation overgrowth or excessive canopy that may obstruct view of right-of-way;

- damaged or missing line marker(s);

damage to coatings or insulation on aboveground pipe or components.

Terms applicable to this task:

This section intentionally left blank.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Encroachment or unauthorized activity on the right-of-way | Make appropriate notifications and follow operator’s procedures. |
| Damaged or missing signs or markers | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Discovery of unintentionally exposed pipeline. | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Dead vegetation, frost ball, vapor cloud, or other indications of release | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Inadequate pipe support or pipe sag | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Facility or pipeline damage | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Accurately identify the right-of-way to be inspected from alignment sheets, GPS, and pipeline maps. | Ensures that the correct line is being inspected and dictates the mode of inspection (aerial patrol, walking the right-of-way, and vehicle patrol). |
| 2 | Perform the visual inspection/patrol of the right-of-way. | Make proper notification of any abnormal conditions according to the operator’s procedures. |
| 3 | Report right-of-way inspection results. | Follow the operator’s procedures. |
| 4 | Complete required documentation. | Follow the operator’s procedures for appropriate documentation, notification protocol, and actions required. |