Task 16.1—Inspect Navigable Waterway Crossing

1.0	Task Description

This task includes the locating and inspection of the below surface pipeline crossings of navigable waterways. Activities would include determining the pipeline depth of cover within the navigable waterway and denoting any other conditions that may result in damage to the pipeline. The activities of this task apply only to the area of the pipeline that is below the water surface.

This task begins after locating the pipeline at water edge of each bank or shoreline and ends when the inspection of conditions and determination of depth of cover of the pipeline that is within the navigable waterway is documented.

The performance of this covered task may require the performance of other covered tasks such as:

Locate Line (reference Task 14.1).

This task does not include but may lead to the performance of other covered tasks such as:

Perform Visual Inspection of Surface Conditions of Right-of-way (reference Task 15.1).

2.0	Knowledge Component

The purpose of this task is to determine pipeline depth of cover, potential pipe exposures, and any other conditions that may result in damage to the pipe within the navigable waterway.

An individual performing this task shall have knowledge of:

specialized locating equipment to determine depth of cover.

Conditions that could impact the safety or integrity of the pipeline include the following:

vessel anchored over the pipeline;

visible sheen or other indications of product release;

waterway bank erosion;

debris lodged against pipeline;

pipe movement or suspended pipeline.

Terms applicable to this task:

depth of cover

The vertical distance from the top of pipe to the soil or water sediment interface.

navigable waterway

A waterway where substantial likelihood of commercial navigation exists.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Response |
| --- | --- |
| Indications of a leak (e.g., bubbles, sheen on water) | Make appropriate notifications and follow the operator’s procedures. Complete other actions, including documentation, as required. |
| Unauthorized vessels above right-of-way (e.g., dredging equipment) | Make appropriate notifications and follow the operator’s procedures. Complete other actions, including documentation, as required. |
| Damaged or missing signs or markers | Make appropriate notifications and follow the operator’s procedures. Complete other actions, including documentation, as required. |
| Discovery of unintentionally exposed pipeline or inadequate depth of cover | Make appropriate notifications and follow the operator’s procedures. Complete other actions, including documentation, as required. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Locate pipeline adjacent to navigable waterway crossing. | This step establishes a reference point for the inspection. The method of locating the line may vary depending on the waterway being inspected. Line locating is a separate covered task (Task 14.1). |
| 2 | Determine the depth of the submerged pipeline by using a probe or specialized electronic equipment. | Determines the amount of cover and verifies the condition of the crossing. |
| 3 | Inspect the conditions of the underwater right-of-way. | Make proper notification of any abnormal conditions according to the operator’s procedures. |
| 4 | Complete required documentation. | Follow the operator’s procedures for appropriate documentation, notification protocol, and actions required. |