Task 7.4—Prepare Surface for Coating by Abrasive Blasting Methods Other Than Water

1.0	Task Description

This task consists of preparing the surface by abrasive blasting other than water.

The task begins with the identification of the area to be prepared and with equipment setup. This task ends when all required information has been documented per the operator’s procedure.

The performance of this covered task may require the performance of other covered tasks such as:

Inspect the Condition of External Coating on Buried or Submerged Pipe (reference Task 5.3);

Visual Inspection of Atmospheric Coating (reference Task 7.1);

Perform Coating Inspection (reference Task 7.7).

This task does not include but may lead to the performance of other covered tasks such as:

- Apply Coating Using Hand Application Methods (reference Task 7.5);

- Apply Coating Using Spray Applications (reference Task 7.6).

2.0	Knowledge Component

The purpose of this task is to prepare surface for coating.

An individual performing this task shall have knowledge of:

- equipment setup logistics and possible environmental concerns associated with the handling and disposal of spent blast media and coating material;

- coating removal, cleaning, and preparation of pipe to accept the coating application;

- hazards related to high-pressure discharge;

- coating manufacturer’s specifications for application or repairs;

- safety and environmental procedures associated with the handling and disposal of spent blast media and coating material;

- surface preparation standards for abrasive blast cleaning;

- types of abrasive blasting including, but not limited to, the following:

  - grit/shot blast;

  - walnut shell;

  - aluminum oxide;

  - crushed slag;

  - glass bead;

  - soda blast.

Terms applicable to this task:

surface profile

The irregular peak and valley profile on a bare surface that can result from operations such as abrasive blast cleaning or power tool cleaning; also known as anchor profile.

Abnormal operating conditions (AOCs) associated with the performance of this task include the following:

| AOC Recognition | AOC Reaction |
| --- | --- |
| Discovery of damage (e.g. mechanical damage or corrosion) on pipeline facilities or components. | Make appropriate notifications according to the operator’s procedures. Complete other actions, including documentation, as required. |
| Unexpected hazardous product encountered when preparing surfaces for coating. | Stop task activities, eliminate ignition source(s), and notify appropriate personnel. |

3.0	Skill Component

To demonstrate proficiency of this task, an individual shall perform the following steps:

| Step | Action | Explanation |
| --- | --- | --- |
| 1 | Identify the area to be abrasive blasted. | This step is necessary to determine where to set up equipment. |
| 2 | Set up equipment logistically per job requirements. | This step confirms safe and efficient operations. |
| 3 | Remove contaminants from specified area, if present. | This step is necessary to avoid grease or oil causing contamination of the tools and surface. NOTE Appropriate personal protective equipment should be utilized to provide protection from contact or injuries from solvents/ detergents, dust, projectiles, and hand/power tools. |
| 4 | Remove existing coating, mill scale, rust, or weld slag, etc., to achieve specified profile. | Address hazardous coatings (e.g. containing asbestos or lead) in accordance with regulatory requirements and the operator’s procedures. NOTE If coating to be applied meets existing coating, the transition may be made by feathering the existing coating at the interface. |
| 5 | Verify the surface profile is met according to the coating specification. | Ensure that surface preparation meets the operator’s standards or specifications. — |
| 6 | Make notifications per the operator’s procedures. | Follow the operator’s policies/procedures for appropriate documentation, notification protocol, and actions required. |